CAMELS-COL: hydrometeorological time series and catchment attributes for 346 catchments in Colombia
(https://doi.org/10.5281/zenodo.15554735)

David A. Jimenez 1,2, Julian E. Meneses 2, Pedro H. Bernardes Solha1, Álvaro Avila Diaz2, Benjamin Quesada2, Bruno Melo Brentan1, André Ferreira Rodrigues 1.

- Hydraulic and Water Resources Department, Federal University of Minas Gerais UFMG, 31270-901, MG, Belo Horizonte, Brazil.

- Earth System Science Program, Faculty of Natural Sciences, Universidad del Rosario, Bogotá D.C., Colombia.

# Licence

Creative Commons Attribution 4.0 International License (CC BY 4.0)

# Citation

When using the data please cite:

Jimenez, D. A., Meneses, J. E., Solha, P. H. B., Avila Diaz, A., Quesada, B., Brentan, B. M., & Rodrigues, A. F. (2025). CAMELS-COL: hydrometeorological time series and catchment attributes for 347 catchments in Colombia [Data set]. Zenodo. https://doi.org/10.5281/zenodo.15554735

# Table of contents

1.	Licence	1

2.	Citation	1

Table of contents	1

3.	Data and file description	1

3.1.	Data processing	5

3.2.	Description of data tables of shapefiles	7

3.2.1.	Boundary files	7

4.	References	8

# Data and file description

This is the CAMELS-COL dataset (Catchment Attributes and Meteorology for Large-sample Studies – Colombia). The dataset comprises the following subsets:

- Boundary: This subset includes two files. The first contains the boundaries of the 346 catchments covered by CAMELS-COL, while the second shows the locations of the streamflow gauging stations managed by IDEAM. The subset is provided in the file 0_Boundary.zip.

- Catchment information: This subset content the file Catchment information.csv  that presents the main information of the catchments included in CAMELS-COL (See Table 1). The subset is provided in file 1_Catchment_information.zip.

Table 1 Information presented in the “Catchment information” subset for the 346 catchments included in

CAMELS-COL.

| Attribute class | Description | Units |
| --- | --- | --- |
| gauge_id | Catchment identifier (IDEAM code of streamflow) | - |
| gauge_department | Department of Colombia where the station is located | - |
| gauge_lat | Northing of the streamflow gauge (World Mercator, EPSG:3395) - m | m |
| gauge_lon | Easting of the streamflow gauge (World Mercator, EPSG:3395) - m | m |
| elev_gauge | Gauge streamflow elevation (meters above sea level) -m.a.s.l. | m |
| gauge_star | Start date of the data record | dd/mm/yyyy |
| gauge_end | End date of the data record | dd/mm/yyyy |
| gauge_n | Number of records | Number |
| gauge_missing | Number of missing values between the start and end dates | % |

- Physiographic characteristics: This subset content the file Physiograpic_characteristics.csv that presents the main physiographic characteristic of the catchment included in CAMELS-COL (See Table 2).  The subset is provided in file 1_Catchment_information.zip.

Table 2 Information presented in the “Physiographic Characteristics” file for the 346 catchments included in

CAMELS-COL.

| Attribute class | Decription | units |
| --- | --- | --- |
| gauge_id | Catchment identifier (IDEAM code of streamflow) | - |
| area | Catchment area | km² |
| perimeter | Catchment perimeter | km |
| gravelius_index | Gravelius compactness index of catchment | - |
| factor_form | Form factor of catchment | - |
| maximum_ele | Maximum elevation of the basin | m |
| mean_ele | Mean elevation of the basin | m |
| minimum_ele | Minimum elevation of the basin | m |
| streng_chanel | Streng of the main chanel | km |
| equi_slope | Equivalent slope of main chanel | m/m |
| cn_catchment | Mean curve number of Soil Conservation Service (SCS) method. | - |
| tc_kirpich | Concentration time by Kirpich Equation | hours |
| tc_chow | Concentration time by Ven te Chow Equation | hours |
| tc_Johnstone | Concentration time by Johnstone Equation | hours |
| tc_Engi_Corps | Concentration time by Corps Engineers Equation | hours |
| Tc_representative | Representative concentration time adopted following the criteria presented in the manuscript. | hours |

- Hydrometeorological data: This subset contains 346 files with hydrometeorological data for each catchment included in CAMELS-COL. Each file presents the information described in Table 3. The time range of the data varies according to the availability of streamflow records. The subset is provided in file 3_Hydrometeorological_data.zip.

Table 3 Information contained in each file of the “Hydrometeorological Data” subset – CAMELS-COL.

| Attribute class | Decription | units |
| --- | --- | --- |
| gauge_id | Catchment identifier (IDEAM code of streamflow) | - |
| pr | Daily precipitation of catchment | mm/dia |
| poten_evapo | Daily evapotranspiration of catchment | mm/dia |
| t_max | Daily maximum temperature of catchment | ºC |
| t_mean | Daily mean temperature of catchment | ºC |
| t_min | Daily minimum temperature of catchment | ºC |
| streamflow | Mean daily streamflow of catchment | m3/s |

- Climatic indices: This subset content the file Climatic_indices.csv that presents the main Climatic indices estimated for the catchment included in CAMELS-COL (See Table 4).  The subset is provided in file 4_Climatic_indices.zip.

Table 4 Information presented in the “Climatic_indices” file for the 346 catchments included in CAMELS-COL.

| Attribute class | Decription | units |
| --- | --- | --- |
| gauge_id | Catchment identifier (IDEAM code of streamflow) | - |
| aridity | Aridity, computed as the ratio of mean ETo to mean precipitation | - |
| high_prec_freq | Frequency of high precipitation days (≥ 5 times the mean daily precipitation) | days yr-1 |
| high_prec_dur | Average duration of high precipitation events (number of consecutive days ≥ 5 times the mean daily precipitation) | days |
| low_prec_freq | Frequency of dry days (< 1 mm day-1) | days yr-1 |
| low_prec_dur | Average duration of dry precipitation events (number of consecutive dry days) | days |

- Hydrological signatures: This subset content the file Hydrological_signatures.csv that presents the main Hydrological signatures estimated for the catchment included in CAMELS-COL (SeeTable 5).  The subset is provided in file 5_Hydrological_signatures.zip.

Table 5 Information presented in the “Hydrological_signatures” file for the 346 catchments included in CAMELS-COL.

| Attribute class | Decription | units |
| --- | --- | --- |
| gauge_id | Catchment identifier (IDEAM code of streamflow) | - |
| q_mean | Mean daily discharge | mm day-1 |
| runoff_ratio | Runoff ratio, computed as the ratio of mean daily discharge to mean daily precipitation | - |
| stream_elas | Streamflow precipitation elasticity (i.e., the sensitivity of streamflow to changes in precipitation at the annual timescale, using the mean daily discharge as reference). See equation 7 in Sankarasubramanian et al. (2001), with the last element being P/Q not Q/P | - |
| slope_fdc | Slope of the flow duration curve between the log-transformed 33rd and 66th streamflow percentiles | - |
| baseflow_index | Baseflow index, computed as the ratio of mean daily baseflow to mean daily discharge, with the hydrograph separation performed using the Ladson et al. (2013) digital filter | - |
| Q5 | 5% flow quantile (low flow) | mm day-1 |
| Q95 | 95% flow quantile (high flow) | mm day-1 |
| high_q_freq | Frequency of high-flow days (> 9 times the median daily flow) | days yr-1 |
| high_q_dur | Average duration of high-flow events (number of consecutive days > 9 times the median daily flow) | days |
| low_q_freq | Frequency of low-flow days (< 0.2 times the mean daily flow) | days yr-1 |
| low_q_dur | Average duration of low-flow days (number of consecutive days < 0.2 times the mean daily flow) | days |

- Land Cover characteristics: This subset contains the file Land_cover_characteristics. csv, which presents, for each basin, the percentage of area covered by the main land cover types identified in the MapBiomas-Colombia project (MapBiomas, 2022) (see Table 6). The subset is provided in the file 6_Land_cover_characteristics.zip.

Table 6 Information presented in the “Land_cover_characteristics” file for the 346 catchments included in CAMELS-COL.

| Attribute class | Decription |
| --- | --- |
| gauge_id | Catchment identifier (IDEAM code of streamflow) |
| forest_perc | Percentage of catchment covered by forest formation |
| nat_non_forest_form_perc | Percentage of catchment covered by natural non forest formation |
| agricul_livestock_perc | Percentage of catchment covered by agriculture and livestock area |
| non_vegeted_perc | Percentage of catchment covered by non-vegetated area |
| water_bodies_perc | Percentage of catchment covered by water body |
| non_identi_land_perc | Percentage of catchment covered by non-identified land cover |

- Geologic characteristics: This subset contains the file Geologic_characteristics. csv, which presents, for each basin, the percentage of the different geological classes present in the basin (see Table 7). The subset is provided in the file 7_Geologic_characteristics.zip.

Table 7 Information presented in the “Geologic_characteristics.xlsx” file for the 346 catchments included in CAMELS-COL.

| Attribute class | Decription |
| --- | --- |
| gauge_id | Catchment identifier (IDEAM code of streamflow) |
| unconso_depo_perc | Percentage of catchment covered by unconsolidated deposit |
| hypabyssal_rock_perc | Percentage of catchment covered by hypabyssal rock |
| metamor_rock_perc | Percentage of catchment covered by metamorphic rock |
| plutonic_rock_perc | Percentage of catchment covered by plutonic rock |
| sedimen_rock_perc | Percentage of catchment covered by sedimentary rock |
| volcaniclastic_rock_perc | Percentage of catchment covered by volcaniclastic rock |
| volcanic_rock_perc | Percentage of catchment covered by volcanic rock |

- Soil characteristics: This subset contains the file Soil characteristics. csv, which presents, for each basin, the percentage of the different geological classes present in the basin (see Table 8). The subset is provided in the file 8_Soil_characteristics.zip.

Table 8 Information presented in the “Soil characteristics.xlsx” file for the 346 catchments included in CAMELS-COL.

| Attribute class | Decription |
| --- | --- |
| gauge_id | Catchment identifier (IDEAM code of streamflow) |
| inceptisols_perc | Percentage of catchment covered by inceptisols |
| entisols_perc | Percentage of catchment covered by entisols |
| andisols_perc | Percentage of catchment covered by andisols |
| urban_perc | Percentage of catchment covered by urban areas |
| alfisols_perc | Percentage of catchment covered by alfisols |
| vertisols_perc | Percentage of catchment covered by vertisols |
| aridisols_perc | Percentage of catchment covered by aridisols |
| water_bodies_perc | Percentage of catchment covered by water bodies |
| ultisols_perc | Percentage of catchment covered by ultisols |
| spodosols_perc | Percentage of catchment covered by spodosols |
| coal_mine_pit_perc | Percentage of catchment covered by coal mine pit |
| histosols_perc | Percentage of catchment covered by histosols |
| oxisols_perc | Percentage of catchment covered by oxisols |
| eroded_misce_perc | Percentage of catchment covered by eroded miscellaneous |
| rocky_misce_perc | Percentage of catchment covered by rocky miscellaneous |
| mollisols_perc | Percentage of catchment covered by mollisols |
| perpetual_snow_perc | Percentage of catchment covered by perpetual snow |

- Land use Capability: This subset contains the file Land_use_capability_R2.csv, which presents, for each basin, the percentage of the different land use capabilities present in the basin (see Table 9). The subset is provided in the file 9_Land use Capability.zip.

Table 9 Information presented in the “Land_use_capability_R2.csv” file for the 346 catchments included in CAMELS-COL.

| Attribute class | Decription |
| --- | --- |
| gauge_id | Catchment identifier (IDEAM code of streamflow) |
| class_1 | Percentage of catchment area classified as Land Use Capability Class 1 |
| class_2 | Percentage of catchment area classified as Land Use Capability Class 2 |
| class_3 | Percentage of catchment area classified as Land Use Capability Class 3 |
| class_4 | Percentage of catchment area classified as Land Use Capability Class 4 |
| class_5 | Percentage of catchment area classified as Land Use Capability Class 5 |
| class_6 | Percentage of catchment area classified as Land Use Capability Class 6 |
| class_7 | Percentage of catchment area classified as Land Use Capability Class 7 |
| class_8 | Percentage of catchment area classified as Land Use Capability Class 8 |
| Urban_area | Percentage of catchment area classified as Land Use Capability Urban area |
| No_classified | Percentage of catchment no classified |

- 10_Drainage_Network: This subset includes one file contains the Drainage Network of the 346 catchments covered by CAMELS-COL. The subset is provided in the file 10_Drainage_Network.zip.

This dataset follows the same standards as other CAMELS datasets, including those for the United States (https://doi.org/10.5194/hess-21-5293-2017), Chile (https://doi.org/10.5194/hess-22-5817-2018), Great Britain (https://doi.org/10.5194/essd-2020-49) and Brazil (https://doi.org/10.5194/essd-12-2075-2020).

## Data processing

Streamflow series

Daily streamflow records were retrieved in February 2023 from the DHIME platform of the Institute of Hydrology, Meteorology and Environmental Studies of Colombia (IDEAM) (access:  http://dhime.ideam.gov.co/atencionciudadano/ ). Following the criteria adopted in other CAMELS datasets (e.g., CAMELS-CL, CAMELS-IND, and CAMELS-DE), only gauging stations with at least 10 years of data available since 1980 were selected. To ensure completeness and reliability, only stations with a minimum data availability of 70% were retained.

Watershed Delineation and Validation

For each selected station, the contributing drainage area was delineated using a 30-meter resolution Digital Elevation Model (DEM) provided by the Instituto Geográfico Agustín Codazzi of Colombia - IGAC (accessible at: Modelo Digital de Elevación. SRTM 30 Metros. Colombia. Año 2011 - Colombia en mapas). This dataset was chosen for its official status and comprehensive coverage, providing a detailed and authoritative representation of soil types across the Colombian territory . The delineation process was automated using WhiteboxTools (Lindsay, 2016), with each streamflow stations serving as the outlet point. Following delineation, the consistency of each basin was manually verified. Basins exhibiting inconsistencies, caused by limitations of the DEM—were excluded from the dataset. As a result, a total of 346 basins were retained.

Climatic Variables

Daily precipitation data were obtained from the CHIRPSv2 dataset (Funk et al., 2015)  for the period 1981–2022. This dataset has a spatial resolution of 0.05° and is available at: https://www.chc.ucsb.edu/data/chirps. Maximum and minimum air temperatures (Tmax and Tmin) were retrieved from the MSWX dataset ( Beck et al., 2022), which has a spatial resolution of 0.1° and is accessible at www.gloh2o.org/mswx. Potential evapotranspiration (ETo) was estimated using the Hargreaves method, with extraterrestrial radiation (Ra) calculated based on latitude and Julian day, following FAO guidelines. For each basin, average daily values of precipitation, temperature, and ETo were computed as the arithmetic mean of all pixel values within its boundaries. No spatial interpolation was applied in order to preserve the original resolution and structure of the gridded datasets.

Geological Information

Geological characteristics were derived from the 2023 Geological Map of Colombia, prepared by Gómez et al. (2023). This dataset was chosen for its official status and comprehensive coverage, providing a detailed and authoritative representation of soil types across the Colombian territory. The lithological composition of each basin was quantified by intersecting the basin polygons with the geological map.

Land Cover Classification

Land cover information for each basin was obtained from the 2022 edition of the MapBiomas Colombia dataset, available at https://colombia.mapbiomas.org/segunda-coleccion-de-mapbiomas-colombia. This dataset is derived from Landsat imagery at a 30-meter spatial resolution. Raster values were intersected with basin polygons to calculate the proportion of each land cover type.

Soil information

Soil characteristics for each basin were derived from the Soil Maps of the Colombian Territory at a 1:100,000 scale, published by the Agustín Codazzi Geographic Institute (IGAC) and available through the Colombian Open Data portal at https://geoportal.igac.gov.co/contenido/datos-abiertos-agrologia. This dataset was chosen for its official status and comprehensive coverage, providing a detailed and authoritative representation of soil types across the Colombian territory. Basin polygons were intersected with the soil map to extract the dominant soil types and their respective proportions within each drainage area.

Land Use capability

Lan use capability characteristics for each basin were derived from the Capacidad de uso de las tierras. República de Colombia. Escala 1:100.000,  published by the Agustín Codazzi Geographic Institute (IGAC) and available through the Colombian Open Data portal at  https://www.colombiaenmapas.gov.co/?u=0&t=43&servicio=1776. This dataset was chosen for its official status and comprehensive coverage. Basin polygons were intersected with the Lan use capability  map to extract the dominant Land use capability classes and their respective proportions within each drainage area.

## Description of data tables of shapefiles

### Boundary files

CAMELS_COL_catchments_boundaries.shp

This shapefile presents the catchment boundaries of 346 basins from CAMELS-COL.

- Type of shapefile: Polygon

- CRS: EPSG:3395 - WGS 84 / World Mercator

| Column header | Unit | Description |
| --- | --- | --- |
| IDEAM_CODE | - | Identification code assigned by IDEAM (Institute of Hydrology, Meteorology and Environmental Studies) |

CAMELS_COL_Gauging.shp

This shapefile presents the Gauging stations localization of 346 basins from CAMELS-COL.

- Type of shapefile: Point

- CRS: EPSG:3395 - WGS 84 / World Mercator

| Column header | Unit | Description |
| --- | --- | --- |
| IDEAM_CODE | - | Identification code assigned by IDEAM (Institute of Hydrology, Meteorology and Environmental Studies) |

Streamflow_network.shp

This shapefile presents the streamflow network of 346 basins from CAMELS-COL.

- Type of shapefile: Point

- CRS: EPSG:3395 - WGS 84 / World Mercator

| Column header | Unit | Description |
| --- | --- | --- |
| IDEAM_CODE | - | Identification code assigned by IDEAM (Institute of Hydrology, Meteorology and Environmental Studies) |

# References

Beck, H. E., Pan, M., Dutra, E., & Miralles, D. G. (2022). Global 3-Hourly 0.1° Bias-Corrected Meteorological Data Including Near-Real-Time Updates and Forecast Ensembles.

Funk, C., Peterson, P., Landsfeld, M., Pedreros, D., Verdin, J., Shukla, S., Husak, G., Rowland, J., Harrison, L., Hoell, A., & Michaelsen, J. (2015). The climate hazards infrared precipitation with stations—A new environmental record for monitoring extremes. Scientific Data, 2(1), 150066. https://doi.org/10.1038/sdata.2015.66

Gómez, J., Montes, N. E., & Marín, E. (2023). Mapa Geológico de Colombia 2023. Escala 1:1 500 000 [Mapa]. Servicio Geológico Colombiano.

Lindsay, J.B., 2016. Whitebox GAT: A case study in geomorphometric analysis. Comput. Geosci. 95, 75–84. https://doi.org/10.1016/j.cageo.2016.07.003